# CSC 3101 – Winter 2020

# Lab Assignment 2

- Design a circuit on Logicly for this 8-3 Multiplexer, shown in your textbook. (40 points)

Output (Y)

| S0 X | S1 Y | S2 Z |
| --- | --- | --- |
|  |  |  | A | B | C | D | E | F | G | H | Expression |
| 0 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |
| 0 | 0 | 1 |  | 1 |  |  |  |  |  |  | Z |
| 0 | 1 | 0 |  |  | 1 |  |  |  |  |  | Y |
| 0 | 1 | 1 |  |  |  | 1 |  |  |  |  | Y Z |
| 1 | 0 | 0 |  |  |  |  | 1 |  |  |  | X |
| 1 | 0 | 1 |  |  |  |  |  | 1 |  |  | XZ |
| 1 | 1 | 0 |  |  |  |  |  |  | 1 |  | X Y |
| 1 | 1 | 1 |  |  |  |  |  |  |  | 1 | X Y Z |

4-Bit Fibonacci Numbers

Design a circuit that accepts a 4-bit input and determines if the accepted input is a number from the Fibonacci series or not. If the accepted number is a number in the Fibonacci series, then the light bulb should be “ON” and the 4-bit digit display should show the number. If the accepted number is not a part of the Fibonacci series, then the light bulb should be “OFF” and the digit display should show 0. (60 points)

| S0 A | S1 B | S3 C | S4 D |
| --- | --- | --- | --- |
|  |  |  |  | 0 | 1 | 2 | 3 | 5 | 8 | 13 | Expression |
| 0 | 0 | 0 | 0 | 1 |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 1 |  | 1 |  |  |  |  |  | D |
| 0 | 0 | 1 | 0 |  |  | 1 |  |  |  |  | C |
| 0 | 0 | 1 | 1 |  |  |  | 1 |  |  |  | C D |
| 0 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |
| 0 | 1 | 0 | 1 |  |  |  |  | 1 |  |  | BD |
| 0 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |
| 0 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 |  |  |  |  |  | 1 |  | A |
| 1 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 | 0 |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 | 1 |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 | 1 |  |  |  |  |  |  | 1 | C |
| 1 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |

Equation:

F = !A !B !C !D + !A !B !C D + !A !B C !D + !A !B C D + !A B !C D + A !B !C !D + A B !C D + A B C D

Simplified:

F = !A !B  + B !C D + !B !C !D